THORGUARDIAN TIMER

OPERATING INSTRUCTIONS TSB031705

Setting Time & Day

Press  MENU  &  release. Push  & release the  HOUR,  MINUTE, & DAY key  till each  is correct. Continuously holding down the key will advance each item at a rapid rate. When complete, press MENU & release.

Setting  Month , Date & Year

Push & release the MONTH, DATE, & YEAR, key until

each Is correct. When complete, press MENU.

Set Alarm Program

This is a (7) seven event  timer. You can select an alarm time Mon – Sun, Mon – Fri, Sat & Sun,  and any specific day of the week. Multiple alarm times can be set to occur

within the same day.

Using the DAY key, select the day interval. Continuously holding  the  key  will  allow  for  self-advance.  Using the HOUR & MINUTE key, select the time the alarm will turn on.

To  delete  a  specific  alarm  cycle ,  press  the DAY key

until the display indicates - -. - - for the “time on”. Then

press ENTER.

Using  the  HOUR  &  MINUTE  key,  select the time that

the alarm will be inactive.

When complete, press the ENTER key.

Continue  this  process  to  enter  additional alarm cycles.

When finished, press the MENU key to exit.

Alarm Mode

By pressing the ALARM MODE key, the (3) different timer

operating modes can be selected:

(Pgm-Aut) ….  The timer is executing the program you

have entered.

(Man-Off)…… Timer overrides the program & disables the horns, strobe  & operational light.

(Man – On)…. Timer overrides the program & enables the horns, strobe & operational light.

1193 Sawgrass Corporate Parkway Sunrise, Florida 33323  Tel (954) 835-0900  Fax (954) 835 -0808